Overview

Quick Details

Classification: Chemical Auxiliary Agent

CAS No.: 8001-54-5

Other Names: BKC

MF: C21H38CIN

EINECS No.: 264-151-6

Purity: 80% 50%

Place of Origin: China

Type: Adsorbent

Adsorbent Variety: Molecular Sieve

Usage: Surfactants, Water Treatment Chemicals, Other

Packaging & Delivery

Packaging Details: Package:200KG/drum;16MT/20'FCL

Product Introduction

### Disinfectant Benzalkonium Chloride 

                                                         CAS No.:8001-54-5 
                                          Package:200KG/drum;16MT/20'FCL

Product Name: Benzalkonium Chloride

CAS No.: 8001-54-5 or 63449-41-2, 139-07-1        

Molecular Weight:  340.0 

Molecular Formula:  C21H38NCl        

Assay:50% & 80% 

EINECS No.:205-351-5

Application

1.Oil & gas industry: strong biocide and algicide ability, to prevent pipe from being blocked and rusted

2.Water treatment : Used as bactericide, kill green,black spot and mustard algae;

3.Detergent: raw detergent materious;

Properties:

Benzalkonium Chloride is a kind of cationic surfactant, a nonoxidizing biocide.

It can efficiently kill bacteria and algae ,and con-reproduction and sludge growth. Benzalkonium Chloride also has dispersing and penetrating properties, can penetrate and remove sludge and algae, has advantages of low toxicity, no toxicity accumulation, soluble in water, convenient to use, unaffected by water hardness.

Usage:

As nonoxidizing biocide, the preferred dosage is 50-100mg/L; as sludge remover, 200-300mg/L dosage is preferred, and it can add the appropriate amount of silicone antifoaming agent if needed for this purpose.

This product can be used together with other fungicidal ,such as isothiazolinones, glutaraldegyde, dithionitrile methane for synergism, but cannot be used together with chlorophenols. If sewage is appeared after thrown of this product in circulating cool water, the sewage should be filtered or blown off in time to prevent their deposit in bottom of collecting tank after froth disappearance.

No blending with anion surfactant.

Specifications

| items | index |  |
| --- | --- | --- |
| Appearance | Colorless to yellowish transparent liquid | Colorless to yellowish transparent liquid |
| Active content % | 48-52 | 78-82 |
| Amine salt % | 2.0 max | 2.0 max |
| pH(1% water solution) | 6.0~8.0(origin) | 6.0-8.0 |

Product Avantages

1.Benzalkonium chloramine owns the broad spectrum, highly effect to be sterilization and algicide,specially for killing the sulfate-reducing bacteria.

2.Soluble in water, easy to use, free from the influence of water hardness, and a strong sticky-mud exfoliation.

3.Low toxicity and no toxicity accumulation, unlike chlorine causes secondary pollution, as it contains residual chlorine if used in water treatment.

4.In the system which can not control if use chlorine, Benzalkonium chloride can effectively control the growth of algal blooms and slime.

5.Even under the different range of PH value, Benzalkonium chloride has strong biocide ability, as it can be used both in acidic and alkaline conditions.

6. Owning dispersion and penetration effect, BKC can penetrate and remove slime and peel attached algae .It also has oil degreasing and cleaning ability.

Package and Storage

Package

200L plastic drum or IBC(1000L) according to customers' requirements.

Storge

It can store for two years in shady room and dry place.

Safety Protection

A little smell of almond, no visible stimulation to skin. Flush in water immediately if contact.